BE LAB TASK # 12/13

Name: Shahmeer khan.

Class ID: 106293.

Student ID: 12113.

Topic: Non-Inverting Voltage Amplifier.

Objectives:

In this exercise, the performance of the non-inverting voltage amplifier will be examined. The investigation will include the effect of feedback resistors on setting voltage gain, stability of gain with differing op amps, and input impedance.

Task (According to Sir):

- Calculations From Voltage 0.3 V:

- Theoretical Av:

- Since Formula:

- Av = 1+Rf / Rin.

Av = 1 + 2k / 1k = 3

Av = 1 + 4.7k / 1k = 5.7

Av = 1 + 10k / 1k = 11

Av = 1 + 22k / 1k = 23

Av = 1 + 33k / 1k = 34

Av = 1 + 47k / 1k = 48

- Experimental Av:

- Since Formula:

- Av = Vout / Vin

Av = 0.8999973 V / 0.3 V = 2.999991

Av = 1.71 V / 0.3 V = 5.7

Av = 3.3 V / 0.3 V = 11

Av = 6.898 V / 0.3 V = 22.99

Av = 10.197 V / 0.3 V = 33.99

Av = 14.393V / 0.3 V = 47.97

- Deviation:

AV (Theoretical) – AV (Experimental) x 100 / Av (Theoretical)

3 - 2999991 x 100 / 3 = 0.003

5.7 – 5.7 x 100 / 5.7 = 0

11 – 11 x 100 / 11 = 0

23 – 22.99 x 100 / 23 = 0.04

34 – 33.99 x 100 / 34 = 0.02

48 – 47.97 x 100 / 48 = 0.06

| RF | THEORETICAL AV | V OUT | EXPERIMENTAL AV | % DEVIATION |
| --- | --- | --- | --- | --- |
| 2 k | 3 | 899.973 m Vpk | 2.99 | 0.33 |
| 4.7k | 5.7 | 1.71 Vpk | 5.7 | 0 |
| 10k | 11 | 3.3 Vpk | 11 | 0 |
| 22K | 23 | 6.898 Vpk | 22.99 | 0.04 |
| 33k | 34 | 10.197 Vpk | 33.99 | 0.02 |
| 47k | 48 | 14.393 Vpk | 47.97 | 0.06 |

Screen-shots:

Link:

https://tinyurl.com/yzg2ful6

- Calculations From Voltage 5 V:

- Theoretical Av:

- Since Formula:

- Av = 1+Rf / Rin.

Av = 1 + 2k / 1k = 3

Av = 1 + 4.7k / 1k = 5.7

Av = 1 + 10k / 1k = 11

Av = 1 + 22k / 1k = 23

Av = 1 + 33k / 1k = 34

Av = 1 + 47k / 1k = 48

- Experimental Av:

- Since Formula:

- Av = Vout / Vin

Av = 15V / 5V = 3

Av = 15.043 / 5V = 3.0086

Av = 15.011V/ 5V = 3.0022

Av = 15V / 5V = 3

Av = 15V/5V = 3

Av = 15.063/5V = 3.0126

- Deviation:

AV (Theoretical) – AV (Experimental) x 100 / Av (Theoretical)

3 – 3 x100 / 3 = 0

5.7 – 3.0086 x 100/5.7 = 47.21

11 – 3.0022 x 100/11 = 72.70

23 – 3 x100/23 = 86.95

34 – 3 x100/34 =91.17

48 – 3.0126/48 = 93.72

| RF | Theoretical AV | V out | Experimental AV | Deviation % |
| --- | --- | --- | --- | --- |
| 2k | 3 | 15 Vpk | 3 | 0 |
| 4.7k | 5.7 | 15.043 Vpk | 3.0086 | 47.21 |
| 10k | 11 | 15.011 Vpk | 3.002 | 72.70 |
| 22k | 23 | 15 Vpk | 3 | 86.95 |
| 33k | 34 | 15 Vpk | 3 | 91.17 |
| 47k | 48 | 15.062 Vpk | 3.012 | 93.72 |

Screen-shots:

Link:

https://tinyurl.com/ydlgqgsk